# Wergonic

Device Manager

Quick Start Guide

A simple guide to set up and use your Wergonic posture feedback device

# What is Wergonic?

Wergonic is a wearable device that helps you maintain good posture. It monitors your arm or trunk position and gently vibrates when you exceed safe angles.

The device provides:

- Real-time angle monitoring

- Vibration alerts when posture thresholds are exceeded

- Session data logging for review

# Getting Started

## What You Need

- A Wergonic device (charged and turned on)

- A computer or phone with Bluetooth

- Google Chrome or Microsoft Edge browser

# Step 1: Connect Your Device

- Turn on your Wergonic device

- Open the Wergonic Device Manager in your browser

- Click "Connect ARM" or "Connect TRUNK" depending on where you'll wear the device

- Select your device (named "Wergonic-XX") from the popup list

- Wait for the green dot to appear - you're connected!

Tip: You can connect two devices at once - one for ARM and one for TRUNK.

# Step 2: Adjust Settings (Optional)

Before calibrating, you can adjust these settings:

## Vibration Intensity

Choose how strong the vibration alerts feel: Weak, Medium, or Strong. You can change this anytime, even during a session.

## Angle Thresholds

Set the angles that trigger alerts:

| Zone | Default (ARM) | Default (TRUNK) |
| --- | --- | --- |
| Yellow (Warning) | 30° | 20° |
| Red (Alert) | 60° | 45° |

Use the sliders to adjust thresholds, then click "Apply".

# Step 3: Calibrate

This is the most important step!

Calibration tells the device what your "neutral" or correct posture looks like. All angle measurements are relative to this position.

- Put on the device and assume your correct/neutral posture

- Stay still in this position

- Click "Calibrate Connected Devices"

- You'll see a confirmation message when done

# Step 4: Start Your Session

After calibration, the device is ready! The Live Data panel shows:

- Angle: Your current angle from the calibrated position

- Zone: GREEN (good), YELLOW (warning), or RED (alert)

You can now disconnect!

Once calibrated, the device works independently. You can close the browser or walk away - the device will continue monitoring your posture and providing vibration feedback on its own.

# Understanding the Feedback

| Zone | What It Means | Vibration |
| --- | --- | --- |
| GREEN | Good posture - keep it up! | None |
| YELLOW | Approaching limit - adjust soon | Two short pulses |
| RED | Over limit - correct posture now | Continuous vibration |

# Ending Your Session

When you're done:

- Reconnect to the device (if disconnected)

- Click "Stop & Save Session" to save your data

- Turn off the device

- (Optional) Download session files from the Session Files panel

# Tips for Best Results

- Calibrate at the start of each session while in your ideal posture

- Start with lower thresholds and adjust as needed

- Use medium intensity first, then adjust based on preference

- Make sure the device is securely attached before calibrating

# Need Help?

| Problem | Solution |
| --- | --- |
| Device not showing up | Make sure Bluetooth is on and device is turned on |
| Can't connect | Try refreshing the page and reconnecting |
| No vibration feedback | Check that feedback is enabled and device is calibrated |
| Wrong angle readings | Recalibrate while in your correct posture |

— Enjoy using your Wergonic device! —